# Code Link:

https://wokwi.com/projects/444175681634130945

# Code:

// Week3-Lecture3

// Two Switch Debounce Solution using Timer Interrupt

// Embedded IoT System Fall-2025

// Name: Usama Bin Saeed          Reg#: 23-NTU-CS-1094

#include <Arduino.h>

#define LED_PIN1       4       //Pin of first LED

#define BUTTON_PIN1    32      // Button for first LED

#define LED_PIN2       5       //Pin of second LED

#define BUTTON_PIN2    26      // Button for second LED

#define DEBOUNCE_MS   50         // Debounce time

#define DEBOUNCE_US   (DEBOUNCE_MS * 1000UL)

hw_timer_t* debounceTimer = nullptr;

volatile bool debounceActive = false;   // blocks re-entrant presses while debouncing

// --- One-shot debounce timer ISR ---

// Fires once after DEBOUNCE_MS; confirms press and toggles LED.

void ARDUINO_ISR_ATTR onDebounceTimer() {

// If still pressed after debounce window, treat as a valid press

if (digitalRead(BUTTON_PIN1) == LOW) { // check if button 1 is pressed

digitalWrite(LED_PIN1, !digitalRead(LED_PIN1));   // invert LED state

}else if (digitalRead(BUTTON_PIN2) == LOW){ //check if button 2 is pressed

digitalWrite(LED_PIN2, !digitalRead((LED_PIN2)));

}

debounceActive = false; // allow next press

}

// --- GPIO button ISR ---

// Starts the one-shot debounce timer on a FALLING edge.

void ARDUINO_ISR_ATTR onButtonISR() {

if (!debounceActive) {

debounceActive = true;

// Arm one-shot alarm: autoreload=false, reload_count=0

timerAlarm(debounceTimer, DEBOUNCE_US, false, 0);

}

}

void setup() {

pinMode(LED_PIN1, OUTPUT);

pinMode(LED_PIN2, OUTPUT);

digitalWrite(LED_PIN1, LOW);

digitalWrite(LED_PIN2, LOW);

// Button to GND with internal pull-up; idle HIGH, pressed LOW

pinMode(BUTTON_PIN1, INPUT_PULLUP);

pinMode(BUTTON_PIN2, INPUT_PULLUP);

attachInterrupt(BUTTON_PIN1, onButtonISR, FALLING);

attachInterrupt(BUTTON_PIN2, onButtonISR, FALLING);

// Create a 1 MHz base timer (1 tick = 1 µs) just for debounce

debounceTimer = timerBegin(1000000);

timerAttachInterrupt(debounceTimer, &onDebounceTimer);

// No alarm yet; we arm it on first button edge

}

void loop() {

// Nothing to do. Interrupts handle everything.

}

# Output:

Purple light is on when BUTTON 1 is pressed.

Red light is on when BUTTON 2 is pressed.